Carson City Community Wildfire Protection Plan

Appendix C: Southwest

August 2023

# Appendix C: Reports for Southwest

## Carson Colony

|  |  | Community Wildfire Risk Assessment. |  |
| --- | --- | --- | --- |
|  |  | Total Assessed Rating. |  |  |  |  |  |
|  |  | 98 - Moderate. |  |  |  |  |  |
|  |  | Suppression Rating. |  |  |  |  |
|  |  | Low Hazard. |  |  |  |
|  |  | Surrounding Environment Rating. |  |  |  |  |
|  |  | High Hazard. |  |  |  |
|  |  | Structures Rating. |  |  |  |  |
|  |  | Moderate Hazard. |  |  |  |
|  |  | Fire Protection District. |  |  |  |  |
|  |  | Carson City Fire. |  |  |  |
|  |  | Fireshed(s). |  |  |  |  |
|  | Carson Valley-Carson River. |  |  |  |
|  | Community Information. |  |  |  |  |
|  |  | Latitude. | 39° 8' 22". |  |  |  |
|  |  | Longitude. | -119° 46' 33". |  |  |  |
|  |  | Dwelling Units. | 100. |  |  |  |
|  |  | Size. | 180.38 acres. |  |  |  |
|  |  | Community Type. | Residential/Commercial. |  |  |  |
|  | Assessed By:. | Scott OBrien. |  |  |  |
|  |  | Assessment Date:. | 03-27-2023. |  |  |  |

|  | SUPPRESSION ASSESSMENT.. Ingress and Egress.. → 2 or more roads in/out with NO response/evacuation complexity... 2 or more roads in/out with SLIGHT response/evacuation complexity... 2 or more roads in/out with MODERATE/HIGH response/evacuation complexity... One road in and out (entrance and exit is the same).... Recommended Mitigation Strategies.. ◻.; Keep community ingress/egress open and maintained (cleared of vegetation)... ◻.; Develop community plan for evacuation routes, safe zones, staging areas... ◻.; If community is gated, develop evacuation plan and ensure emergency responder access... ◻.; Ensure residents know their closest exit in case of emergency... ◻.; Ask Local Fire Department about Ready, Set, Go!...... Road Width.. → Road width is > 24 feet... Road width is > 20 feet and < 24 feet... Road width is < 20 feet.... Recommended Mitigation Strategies.. ◻.; Keep shoulders of road clear for emergency vehicle use whenever possible...... Road Accessibility.. → Surfaced road... Non-surfaced road, grade less than or equal to 5%... Non-surfaced road, grade greater than 5%... Non-maintained dirt road.... Recommended Mitigation Strategies.. ◻.; Ensure that road maintenance plan is in place...... |
| --- | --- |

|  | Secondary Road Terminus.. → Roads ends in a cul-de-sac, diameter > 100 feet... Roads ends in a cul-de-sac, diameter < 100 feet... Dead end roads <200 feet long... Dead end roads >200 feet long.... Recommended Mitigation Strategies.. ◻.; Maintain unobstructed access into cul-de-sacs... ◻.; Ensure cul-de-sacs are free of vehicles and/or other items...... Street Signs.. → Present throughout, lettering 4 inches high, non-flammable and reflective... Inconsistent throughout, lettering 4 inches high, non-flammable and reflective... Present or inconsistent but wooden, non-reflective, or lettering less than 4"... Not present.... Recommended Mitigation Strategies.. ◻.; Keep street signs visible and clear of vegetation and fine fuels...... Driveways.. → Average driveway allows access to homes... Average driveway restricts access to homes.... Recommended Mitigation Strategies.. ◻.; Maintain driveway access and clearance...... Water Supply.. → Pressurized hydrants spaced less than 1000 feet apart... Pressurized hydrants spaced more than 1000 feet apart... Dry Hydrant(s) / Draft available within the community... Other accessible sources within community (pond, lake, etc.)... Water sources located within 4 miles of community (incl heli dip sites)... No water sources in or within 4 miles of the community.... Recommended Mitigation Strategies.. ◻.; Ensure hydrants and water sources are marked, accessible and properly maintained... ◻.; Keep hydrants clear of obstructions and vegetation...... |
| --- | --- |

|  | Geographic Features.. No notable geographical features present to hinder fire suppression... → Suppression efforts hindered by geographical features (e.g. hazardous terrain).... Recommended Mitigation Strategies.. ◻.; Ensure emergency responders are aware of local geographic features that can hinder fire suppression efforts; consider pre-suppression plan...... Local Response Resources.. → 5 mi. or less from Agency with Response Authority (Staffed FD)... 5 mi. or less from Agency with Response Authority (Mixed Staff/VFD)... 5 mi. or less from Agency with Response Authority (VFD)... > 5 mi. from Agency with Response Authority FD.... Recommended Mitigation Strategies.. ◻.; Establish and maintain contact with the closest Fire Department; consider pre-suppression plan... ◻.; Be aware of the importance of early detection and reporting of any emergency...... Community Organization/Governance.. GID present; HAS structure for sustained fire prevention and mitigation... HOA present; HAS structure for sustained fire prevention and mitigation... Municipal govt present; HAS structure for sustained fire prevention and mitigation... GID present; LACKS structure for sustained fire prevention and mitigation... HOA present; LACKS structure for sustained fire prevention and mitigation... Municipal govt present; LACKS structure for sustained fire prevention and mitigation... → Lacks any structure for sustained fire prevention and mitigation.... Recommended Mitigation Strategies.. ◻.; Work with community to become more proactive towards protecting your life and property against wildfires; Become a Firewise USA® Site... ◻.; Host a Community Education Event at least once a year... ◻.; Complete Community Risk Mitigation Project(s) as identified by Community Action Plan... ◻.; Ensure individual homes are ignition-resistant, hardened, and Firewise/Living with Fire concepts are followed...... |
| --- | --- |

|  | SURROUNDING ENVIRONMENT ASSESSMENT.. Predominant Vegetation.. Light (grass)... → Medium (brush)... Heavy (timber, overgrown sage, Pinyon/Juniper with dead/down, etc)... Extreme / Slash (Any Combination of contiguous Light, Medium, Heavy).... Recommended Mitigation Strategies.. ◻.; Consider removal of ladder fuels that allow fire to climb from lower to higher vegetation... ◻.; Trim tree canopies regularly to keep their branches a minimum of 10' from structures and other trees... ◻.; Leave 30 feet between clusters of two to three trees, or 20 feet between individual trees... ◻.; Prune trees 6-10 feet from the ground...... Defensible Space.. > 75% of homes meet criteria in Zone 0, 1 & 2... 50 to 75% of homes meet criteria in Zone 0, 1 & 2... < 50% of homes meet criteria in Zone 0, 1 & 2 - Light fuels amongst structures... → < 50% of homes meet criteria in Zone 0, 1 & 2 - Moderate fuels amongst structures... Fuels heavy/extreme amongst structures & other urban hazards/materials are present.... Recommended Mitigation Strategies.. ◻.; Be aware of the risks from falling embers in relation to nearby fuels and defensible space... ◻.; Mow lawns regularly... ◻.; Water grass, plants, trees and mulch regularly... ◻.; Create a spacing of 30 feet between tree crowns... ◻.; Create a non-combustible area (zone 0) within 5 feet of your home, using non-flammable landscaping materials... ◻.; Remove dead vegetation from under the deck and within 10 feet of the house; stack firewood away from structures... ◻.; ConsiderÂ xeriscaping... ◻.; Plant a mixture of deciduous trees (e.g. oak and maple) and coniferous trees (e.g. pine)... ◻.; Create fuel breaks like driveways and gravel walkways...... |
| --- | --- |

|  | Structure-to-Structure Ignition.. No Possible Structure-to-Structure Ignition... → Possible Structure-to-Structure Ignition.... Recommended Mitigation Strategies.. ◻.; Work with neighbors to remove/prune vegetation between houses to mitigate structure-to-structure ignition risk; consder non-combustible fencing 5 feet from structure... ◻.; Consider use of sprinkler systems to keep vegetation moisture levels up... ◻.; Replace flammable roofs, siding, soffits, etc. with nonflammable when possible...... Slope.. → Slope 0% - 5%... Slope 6 % - 10%... Slope 11% - 30%... Slope > 31%.... Recommended Mitigation Strategies.. ◻.; N/A...... Vegetation on Electric Transmission Lines.. No above ground electric transmission lines present... → Above ground electric transmission lines are maintained... Above ground electric transmission lines are NOT maintained.... Recommended Mitigation Strategies.. ◻.; Know who to call should there be a problem with electric lines in community...... Topographical Features.. No topographical features adversely affect wildland fire behavior... → Topographical features adversely affect wildland fire behavior (box canyons, chimneys, etc.).... Recommended Mitigation Strategies.. ◻.; Maintain situational awareness of fire danger in your area, as local topographical features can adversely affect wildland behavior...... |
| --- | --- |

|  | Adjacency to Wildlands.. Not adjacent to wildlands with accumulated fuels... → Adjacent to wildlands with accumulated fuels.... Recommended Mitigation Strategies.. ◻.; When possible, install firebreaks and reduce fuel loads around community boundary to reduce risk from adjacent wildlands; Work with neighboring land owners...... Undeveloped Lots with Restricted Access and/or Not Maintained.. Fewer than 10% of lots are undeveloped... 10% to 30% of lots are undeveloped... → 31% to 50% of lots are undeveloped... Greater than 51% of lots are undeveloped.... Recommended Mitigation Strategies.. ◻.; Provide Living with Fire/Firewise construction guidelines to developers /owners... ◻.; Consider developing covenant restrictions, if applicable...... |
| --- | --- |

STRUCTURES ASSESSMENT.. Roofing Materials.. → > 75% of homes have metal, tile or class A asphalt or fiberglass shingles... 50 to 75% of homes have metal, tile or class A asphalt or fiberglass shingles... < 50% of homes have metal, tile or class A asphalt or fiberglass shingles.... Recommended Mitigation Strategies.. ◻.; Use fire-resistant roofing material such as metal, tile or Class A shingles... ◻.; Inspect for and address gaps in roofing that can expose roof decking or supports... ◻.; Place angle flashing over openings between the roof decking and fascia board...... Debris on Roof and/or Gutters.. → No... Yes.... Recommended Mitigation Strategies.. ◻.; Clear branch, leaf-litter and other debris from roof and gutters regularly... ◻.; Prune tree limbs away from roof...... Ventilation and Soffits.. → > 75% of homes have non-combustible ventilation soffits with mesh or screening... 50-74% of homes have non-combustible ventilation soffits with mesh or screening... < 50% of homes have non-combustible ventilation soffits with mesh or screening.... Recommended Mitigation Strategies.. ◻.; Clean vents to keep them free of debris, allowing them to keep embers out while allowing air flow for ventilation... ◻.; Enclose or box-in eaves with non-combustible materials such as metal, cement board or stucco... ◻.; Install a 1/8 inch metal screen behind roof vents...... Siding.. > 75% of homes have non-combustible siding... → 50-74% of homes have non-combustible siding... < 50% of homes have non-combustible siding.... Recommended Mitigation Strategies.. ◻.; Keep landscaping materials and vegetation away from combustible siding... ◻.; Create 5-foot non-combustible area (Zone 0) around house... ◻.; Replace with noncombustible siding when possible......

Underskirting.. > 75% of homes have skirting underneath raised floors/decks... 50-74% of homes have skirting underneath... → < 50% of homes have skirting underneath.... Recommended Mitigation Strategies.. ◻.; Remove combustible vegetation and leaf litter... ◻.; Spread gravel or other non-combustible material under the deck... ◻.; Screen in the bottom of the deck with metal 1/8-inch screening... ◻.; Separate wooden fences from the house with a stone or metal barrier...... Wooden Attachments.. → > 75% of homes have NO Wooden Attachments... 50-74% of homes have NO Wooden Attachments... < 50% of homes have NO Wooden Attachments.... Recommended Mitigation Strategies.. ◻.; Maintain debris-free decks (e.g. remove ignitable furniture, planters and covering propane grills, especially during high fire danger periods)... ◻.; Consider disconnecting fences from structures, or replacing materials directly attached to structures with fire resistant materials... ◻.; Be aware that wooden attachments can act as a fuse to the structure...... Building Setback.. → Not applicable... Greater than or equal to 30 feet from slope... Less than 30 feet from slope.... Recommended Mitigation Strategies.. ◻.; N/A...... Propane.. > 30 feet from the house and surrounding vegetation maintained... Fewer than 30 feet from the house and/or surrounding vegetation not maintained... → N/A.... Recommended Mitigation Strategies.. ◻.; N/A......

| Electric Utilities.. Electric Underground... → Electric Overhead drop maintained... Electric Overhead drop not maintained.... Recommended Mitigation Strategies.. ◻.; Â Keep vegetation pruned and mowed around electric cabinets... ◻.; Place non-flammable mulch (rock, stone) around base of electrical cabinets... ◻.; Plant less flammable bushes and shrubs around electrical cabinets...... Non-Combustible Zone 0.. → > 75% of homes/outbuildings have adjacent 5-ft non-combustible zone... 50-74% of homes/outbuildings have adjacent 5-ft non-combustible zone... < 50% of homes/outbuildings have adjacent 5-ft non-combustible zone.... Recommended Mitigation Strategies.. ◻.; Remove flammable materials and Create 5-foot non-combustible area (Zone 0) around house...... |
| --- |
|  |  | COMMENTS. |  |

## Clear Creek

|  |  | Community Wildfire Risk Assessment. |  |
| --- | --- | --- | --- |
|  |  | Total Assessed Rating. |  |  |  |  |  |
|  |  | 181 - High. |  |  |  |  |  |
|  |  | Suppression Rating. |  |  |  |  |
|  |  | Extreme Hazard. |  |  |  |
|  |  | Surrounding Environment Rating. |  |  |  |  |
|  |  | High Hazard. |  |  |  |
|  |  | Structures Rating. |  |  |  |  |
|  |  | High Hazard. |  |  |  |
|  |  | Fire Protection District. |  |  |  |  |
|  |  | Carson City Fire. |  |  |  |
|  |  | Fireshed(s). |  |  |  |  |
|  | Carson Valley-Carson River. |  |  |  |
|  | Community Information. |  |  |  |  |
|  |  | Latitude. | 39° 7' 3". |  |  |  |
|  |  | Longitude. | -119° 49' 51". |  |  |  |
|  |  | Dwelling Units. | 55. |  |  |  |
|  |  | Size. | 405.33 acres. |  |  |  |
|  |  | Community Type. | Residential - Stick-Built. |  |  |  |
|  | Assessed By:. | Scott OBrien. |  |  |  |
|  |  | Assessment Date:. | 04-11-2023. |  |  |  |

|  | SUPPRESSION ASSESSMENT.. Ingress and Egress.. 2 or more roads in/out with NO response/evacuation complexity... 2 or more roads in/out with SLIGHT response/evacuation complexity... 2 or more roads in/out with MODERATE/HIGH response/evacuation complexity... → One road in and out (entrance and exit is the same).... Recommended Mitigation Strategies.. ◻.; Keep community ingress/egress open and maintained (cleared of vegetation, vehicles, and/or any obstructions)... ◻.; Consider developing safety zones and a plan for Shelter-in-Place; consult with Local Fire Department... ◻.; Consider adding a secondary ingress / egress route for use in emergencies...... Road Width.. Road width is > 24 feet... Road width is > 20 feet and < 24 feet... → Road width is < 20 feet.... Recommended Mitigation Strategies.. ◻.; Keep shoulders of road clear for emergency vehicle use at all times... ◻.; Consider providing pull-offs every 100 yards for emergency vehicle use... ◻.; Coordinate with fire department to ensure they are aware of road width limitations... ◻.; Be aware that road width could limit emergency vehicles to brush trucks only...... Road Accessibility.. → Surfaced road... Non-surfaced road, grade less than or equal to 5%... Non-surfaced road, grade greater than 5%... Non-maintained dirt road.... Recommended Mitigation Strategies.. ◻.; Ensure that road maintenance plan is in place...... |
| --- | --- |

|  | Secondary Road Terminus.. Roads ends in a cul-de-sac, diameter > 100 feet... Roads ends in a cul-de-sac, diameter < 100 feet... Dead end roads <200 feet long... → Dead end roads >200 feet long.... Recommended Mitigation Strategies.. ◻.; Ensure emergency responders are aware of dead-end roads; Consider signing all dead ends.... ◻.; If dead-end roads are narrow, restrict access during an emergency...... Street Signs.. → Present throughout, lettering 4 inches high, non-flammable and reflective... Inconsistent throughout, lettering 4 inches high, non-flammable and reflective... Present or inconsistent but wooden, non-reflective, or lettering less than 4"... Not present.... Recommended Mitigation Strategies.. ◻.; Keep street signs visible and clear of vegetation and fine fuels...... Driveways.. Average driveway allows access to homes... → Average driveway restricts access to homes.... Recommended Mitigation Strategies.. ◻.; Improve driveway accessibility where possible... ◻.; Ensure emergency responders are aware of driveway restrictions...... Water Supply.. Pressurized hydrants spaced less than 1000 feet apart... Pressurized hydrants spaced more than 1000 feet apart... Dry Hydrant(s) / Draft available within the community... → Other accessible sources within community (pond, lake, etc.)... Water sources located within 4 miles of community (incl heli dip sites)... No water sources in or within 4 miles of the community.... Recommended Mitigation Strategies.. ◻.; Coordinate with fire department and land owners to train/test use of local water sources (e.g., ponds, lakes)...... |
| --- | --- |

|  | Geographic Features.. No notable geographical features present to hinder fire suppression... → Suppression efforts hindered by geographical features (e.g., hazardous terrain).... Recommended Mitigation Strategies.. ◻.; Ensure emergency responders are aware of local geographic features that can hinder fire suppression efforts; consider pre-suppression plan...... Local Response Resources.. → 5 mi. or less from Agency with Response Authority (Staffed FD)... 5 mi. or less from Agency with Response Authority (Mixed Staff/VFD)... 5 mi. or less from Agency with Response Authority (VFD)... > 5 mi. from Agency with Response Authority FD.... Recommended Mitigation Strategies.. ◻.; Establish and maintain contact with the closest Fire Department; consider pre-suppression plan... ◻.; Be aware of the importance of early detection and reporting of any emergency...... Community Organization/Governance.. GID present; HAS structure for sustained fire prevention and mitigation... HOA present; HAS structure for sustained fire prevention and mitigation... Municipal govt present; HAS structure for sustained fire prevention and mitigation... GID present; LACKS structure for sustained fire prevention and mitigation... → HOA present; LACKS structure for sustained fire prevention and mitigation... Municipal govt present; LACKS structure for sustained fire prevention and mitigation... Lacks any structure for sustained fire prevention and mitigation.... Recommended Mitigation Strategies.. ◻.; Work with community to become more proactive towards protecting your life and property against wildfires; Become a Firewise USA® Site... ◻.; Host a Community Education Event at least once a year; Become a Firewise USA® Site... ◻.; Complete Community Risk Mitigation Project(s) as identified by Community Action Plan...... |
| --- | --- |

|  | SURROUNDING ENVIRONMENT ASSESSMENT.. Predominant Vegetation.. Light (grass)... Medium (brush)... → Heavy (timber, overgrown sage, Pinyon/Juniper with dead/down, etc.)... Extreme / Slash (Any Combination of contiguous Light, Medium, Heavy).... Recommended Mitigation Strategies.. ◻.; Consider removal of ladder fuels that allow fire to climb from lower to higher vegetation... ◻.; Trim tree canopies regularly to keep their branches a minimum of 10' from structures and other trees... ◻.; Leave 30 feet between clusters of two to three trees, or 20 feet between individual trees... ◻.; Prune trees 6-10 feet from the ground...... Defensible Space.. > 75% of homes meet criteria in Zone 0, 1 & 2... 50 to 75% of homes meet criteria in Zone 0, 1 & 2... < 50% of homes meet criteria in Zone 0, 1 & 2 - Light fuels amongst structures... → < 50% of homes meet criteria in Zone 0, 1 & 2 - Moderate fuels amongst structures... Fuels heavy/extreme amongst structures & other urban hazards/materials are present.... Recommended Mitigation Strategies.. ◻.; Be aware of the risks from falling embers in relation to nearby fuels and defensible space... ◻.; Mow lawns regularly... ◻.; Water grass, plants, trees, and mulch regularly... ◻.; Create a spacing of 30 feet between tree crowns... ◻.; Create a non-combustible area (zone 0) within 5 feet of your home, using non-flammable landscaping materials... ◻.; Remove dead vegetation from under the deck and within 10 feet of the house; stack firewood away from structures... ◻.; Consider xeriscaping... ◻.; Plant a mixture of deciduous trees (e.g., oak and maple) and coniferous trees (e.g. pine)... ◻.; Create fuel breaks like driveways and gravel walkways...... |
| --- | --- |

|  | Structure-to-Structure Ignition.. No Possible Structure-to-Structure Ignition... → Possible Structure-to-Structure Ignition.... Recommended Mitigation Strategies.. ◻.; Work with neighbors to remove/prune vegetation between houses to mitigate structure-to-structure ignition risk; consider non-combustible fencing 5 feet from structure... ◻.; Consider use of sprinkler systems to keep vegetation moisture levels up... ◻.; Replace flammable roofs, siding, soffits, etc. with nonflammable when possible...... Slope.. → Slope 0% - 5%... Slope 6 % - 10%... Slope 11% - 30%... Slope > 31%.... Recommended Mitigation Strategies.. ◻.; N/A...... Vegetation on Electric Transmission Lines.. No above ground electric transmission lines present... → Above ground electric transmission lines are maintained... Above ground electric transmission lines are NOT maintained.... Recommended Mitigation Strategies.. ◻.; Know who to call should there be a problem with electric lines in community...... Topographical Features.. No topographical features adversely affect wildland fire behavior... → Topographical features adversely affect wildland fire behavior (box canyons, chimneys, etc.).... Recommended Mitigation Strategies.. ◻.; Maintain situational awareness of fire danger in your area, as local topographical features can adversely affect wildland behavior...... |
| --- | --- |

|  | Adjacency to Wildlands.. Not adjacent to wildlands with accumulated fuels... → Adjacent to wildlands with accumulated fuels.... Recommended Mitigation Strategies.. ◻.; When possible, install firebreaks and reduce fuel loads around community boundary to reduce risk from adjacent wildlands; Work with neighboring land owners...... Undeveloped Lots with Restricted Access and/or Not Maintained.. Fewer than 10% of lots are undeveloped... 10% to 30% of lots are undeveloped... → 31% to 50% of lots are undeveloped... Greater than 51% of lots are undeveloped.... Recommended Mitigation Strategies.. ◻.; Provide Living with Fire/Firewise construction guidelines to developers /owners... ◻.; Consider developing covenant restrictions, if applicable...... |
| --- | --- |

STRUCTURES ASSESSMENT.. Roofing Materials.. → > 75% of homes have metal, tile or class A asphalt or fiberglass shingles... 50 to 75% of homes have metal, tile or class A asphalt or fiberglass shingles... < 50% of homes have metal, tile or class A asphalt or fiberglass shingles.... Recommended Mitigation Strategies.. ◻.; Use fire-resistant roofing material such as metal, tile, or Class A shingles... ◻.; Inspect for and address gaps in roofing that can expose roof decking or supports... ◻.; Place angle flashing over openings between the roof decking and fascia board...... Debris on Roof and/or Gutters.. No... → Yes.... Recommended Mitigation Strategies.. ◻.; Clear branch, leaf-litter and other debris from roof and gutters regularly... ◻.; Prune tree limbs away from roof...... Ventilation and Soffits.. → > 75% of homes have non-combustible ventilation soffits with mesh or screening... 50-74% of homes have non-combustible ventilation soffits with mesh or screening... < 50% of homes have non-combustible ventilation soffits with mesh or screening.... Recommended Mitigation Strategies.. ◻.; Clean vents to keep them free of debris, allowing them to keep embers out while allowing air flow for ventilation... ◻.; Enclose or box-in eaves with non-combustible materials such as metal, cement board or stucco... ◻.; Install a 1/8 inch metal screen behind roof vents...... Siding.. > 75% of homes have non-combustible siding... 50-74% of homes have non-combustible siding... → < 50% of homes have non-combustible siding.... Recommended Mitigation Strategies.. ◻.; Keep landscaping materials and vegetation away from combustible siding... ◻.; Create 5-foot non-combustible area (Zone 0) around house... ◻.; Replace with noncombustible siding when possible......

Underskirting.. > 75% of homes have skirting underneath raised floors/decks... 50-74% of homes have skirting underneath... → < 50% of homes have skirting underneath.... Recommended Mitigation Strategies.. ◻.; Remove combustible vegetation and leaf litter... ◻.; Spread gravel or other non-combustible material under the deck... ◻.; Screen in the bottom of the deck with metal 1/8-inch screening... ◻.; Separate wooden fences from the house with a stone or metal barrier...... Wooden Attachments.. > 75% of homes have NO Wooden Attachments... 50-74% of homes have NO Wooden Attachments... → < 50% of homes have NO Wooden Attachments.... Recommended Mitigation Strategies.. ◻.; Â Maintain debris-free decks (e.g., remove ignitable furniture, planters and covering propane grills, especially during high fire danger periods)... ◻.; Consider disconnecting fences from structures, or replacing materials directly attached to structures with fire resistant materials... ◻.; Be aware that wooden attachments can act as a fuse to the structure...... Building Setback.. → Not applicable... Greater than or equal to 30 feet from slope... Less than 30 feet from slope.... Recommended Mitigation Strategies.. ◻.; N/A...... Propane.. > 30 feet from the house and surrounding vegetation maintained... Fewer than 30 feet from the house and/or surrounding vegetation not maintained... → N/A.... Recommended Mitigation Strategies.. ◻.; N/A......

| Electric Utilities.. → Electric Underground... Electric Overhead drop maintained... Electric Overhead drop not maintained.... Recommended Mitigation Strategies.. ◻.; Keep vegetation pruned and mowed around electric cabinets... ◻.; Place non-flammable materials (rock, stone) around base of electrical cabinets... ◻.; Plant less flammable bushes and shrubs around electrical cabinets...... Non-Combustible Zone 0.. > 75% of homes/outbuildings have adjacent 5-ft non-combustible zone... 50-74% of homes/outbuildings have adjacent 5-ft non-combustible zone... → < 50% of homes/outbuildings have adjacent 5-ft non-combustible zone.... Recommended Mitigation Strategies.. ◻.; N/A...... |
| --- |
|  |  | COMMENTS. |  |

## Costco

|  |  | Community Wildfire Risk Assessment. |  |
| --- | --- | --- | --- |
|  |  | Total Assessed Rating. |  |  |  |  |  |
|  |  | 73 - Moderate. |  |  |  |  |  |
|  |  | Suppression Rating. |  |  |  |  |
|  |  | Low Hazard. |  |  |  |
|  |  | Surrounding Environment Rating. |  |  |  |  |
|  |  | Moderate Hazard. |  |  |  |
|  |  | Structures Rating. |  |  |  |  |
|  |  | Moderate Hazard. |  |  |  |
|  |  | Fire Protection District. |  |  |  |  |
|  |  | Carson City Fire. |  |  |  |
|  |  | Fireshed(s). |  |  |  |  |
|  | Carson Valley-Carson River. |  |  |  |
|  | Community Information. |  |  |  |  |
|  |  | Latitude. | 39° 6' 58". |  |  |  |
|  |  | Longitude. | -119° 46' 37". |  |  |  |
|  |  | Dwelling Units. | 40. |  |  |  |
|  |  | Size. | 79.15 acres. |  |  |  |
|  |  | Community Type. | Residential/Commercial. |  |  |  |
|  | Assessed By:. | Scott OBrien. |  |  |  |
|  |  | Assessment Date:. | 03-27-2023. |  |  |  |

|  | SUPPRESSION ASSESSMENT.. Ingress and Egress.. → 2 or more roads in/out with NO response/evacuation complexity... 2 or more roads in/out with SLIGHT response/evacuation complexity... 2 or more roads in/out with MODERATE/HIGH response/evacuation complexity... One road in and out (entrance and exit is the same).... Recommended Mitigation Strategies.. ◻.; Keep community ingress/egress open and maintained (cleared of vegetation)... ◻.; Develop community plan for evacuation routes, safe zones, staging areas... ◻.; If community is gated, develop evacuation plan, and ensure emergency responder access... ◻.; Ensure residents know their closest exit in case of emergency... ◻.; Ask Local Fire Department about Ready, Set, Go!...... Road Width.. Road width is > 24 feet... → Road width is > 20 feet and < 24 feet... Road width is < 20 feet.... Recommended Mitigation Strategies.. ◻.; Keep shoulders of road clear for emergency vehicle use at all times... ◻.; Consider providing pull-offs every 100 yards...... Road Accessibility.. → Surfaced road... Non-surfaced road, grade less than or equal to 5%... Non-surfaced road, grade greater than 5%... Non-maintained dirt road.... Recommended Mitigation Strategies.. ◻.; Ensure that road maintenance plan is in place...... |
| --- | --- |

|  | Secondary Road Terminus.. → Roads ends in a cul-de-sac, diameter > 100 feet... Roads ends in a cul-de-sac, diameter < 100 feet... Dead end roads <200 feet long... Dead end roads >200 feet long.... Recommended Mitigation Strategies.. ◻.; Maintain unobstructed access into cul-de-sacs... ◻.; Ensure cul-de-sacs are free of vehicles and/or other items...... Street Signs.. → Present throughout, lettering 4 inches high, non-flammable and reflective... Inconsistent throughout, lettering 4 inches high, non-flammable and reflective... Present or inconsistent but wooden, non-reflective, or lettering less than 4"... Not present.... Recommended Mitigation Strategies.. ◻.; Keep street signs visible and clear of vegetation and fine fuels...... Driveways.. → Average driveway allows access to homes... Average driveway restricts access to homes.... Recommended Mitigation Strategies.. ◻.; Maintain driveway access and clearance...... Water Supply.. → Pressurized hydrants spaced less than 1000 feet apart... Pressurized hydrants spaced more than 1000 feet apart... Dry Hydrant(s) / Draft available within the community... Other accessible sources within community (pond, lake, etc.)... Water sources located within 4 miles of community (incl heli dip sites)... No water sources in or within 4 miles of the community.... Recommended Mitigation Strategies.. ◻.; Ensure hydrants and water sources are marked, accessible and properly maintained... ◻.; Keep hydrants clear of obstructions and vegetation...... |
| --- | --- |

|  | Geographic Features.. No notable geographical features present to hinder fire suppression... → Suppression efforts hindered by geographical features (e.g., hazardous terrain).... Recommended Mitigation Strategies.. ◻.; Ensure emergency responders are aware of local geographic features that can hinder fire suppression efforts; consider pre-suppression plan...... Local Response Resources.. → 5 mi. or less from Agency with Response Authority (Staffed FD)... 5 mi. or less from Agency with Response Authority (Mixed Staff/VFD)... 5 mi. or less from Agency with Response Authority (VFD)... > 5 mi. from Agency with Response Authority FD.... Recommended Mitigation Strategies.. ◻.; Establish and maintain contact with the closest Fire Department; consider pre-suppression plan... ◻.; Be aware of the importance of early detection and reporting of any emergency...... Community Organization/Governance.. GID present; HAS structure for sustained fire prevention and mitigation... HOA present; HAS structure for sustained fire prevention and mitigation... → Municipal govt present; HAS structure for sustained fire prevention and mitigation... GID present; LACKS structure for sustained fire prevention and mitigation... HOA present; LACKS structure for sustained fire prevention and mitigation... Municipal govt present; LACKS structure for sustained fire prevention and mitigation... Lacks any structure for sustained fire prevention and mitigation.... Recommended Mitigation Strategies.. ◻.; Host a Community Education Event at least once a year; Become a Firewise USA® Site... ◻.; Complete Community Risk Mitigation Project(s) as identified by Community Action Plan...... |
| --- | --- |

|  | SURROUNDING ENVIRONMENT ASSESSMENT.. Predominant Vegetation.. Light (grass)... → Medium (brush)... Heavy (timber, overgrown sage, Pinyon/Juniper with dead/down, etc.)... Extreme / Slash (Any Combination of contiguous Light, Medium, Heavy).... Recommended Mitigation Strategies.. ◻.; Consider removal of ladder fuels that allow fire to climb from lower to higher vegetation... ◻.; Trim tree canopies regularly to keep their branches a minimum of 10' from structures and other trees... ◻.; Leave 30 feet between clusters of two to three trees, or 20 feet between individual trees... ◻.; Prune trees 6-10 feet from the ground...... Defensible Space.. → > 75% of homes meet criteria in Zone 0, 1 & 2... 50 to 75% of homes meet criteria in Zone 0, 1 & 2... < 50% of homes meet criteria in Zone 0, 1 & 2 - Light fuels amongst structures... < 50% of homes meet criteria in Zone 0, 1 & 2 - Moderate fuels amongst structures... Fuels heavy/extreme amongst structures & other urban hazards/materials are present.... Recommended Mitigation Strategies.. ◻.; Be aware of the risks from falling embers in relation to nearby fuels and defensible space... ◻.; Mow lawns regularly... ◻.; Water grass, plants, trees, and mulch regularly... ◻.; Create a spacing of 30 feet between tree crowns... ◻.; Create a non-combustible area (zone 0) within 5 feet of your home, using non-flammable landscaping materials... ◻.; Remove dead vegetation from under the deck and within 10 feet of the house; stack firewood away from structures... ◻.; Consider xeriscaping...... |
| --- | --- |

|  | Structure-to-Structure Ignition.. No Possible Structure-to-Structure Ignition... → Possible Structure-to-Structure Ignition.... Recommended Mitigation Strategies.. ◻.; Work with neighbors to remove/prune vegetation between houses to mitigate structure-to-structure ignition risk; consider non-combustible fencing 5 feet from structure... ◻.; Consider use of sprinkler systems to keep vegetation moisture levels up... ◻.; Replace flammable roofs, siding, soffits, etc. with nonflammable when possible...... Slope.. → Slope 0% - 5%... Slope 6 % - 10%... Slope 11% - 30%... Slope > 31%.... Recommended Mitigation Strategies.. ◻.; N/A...... Vegetation on Electric Transmission Lines.. No above ground electric transmission lines present... → Above ground electric transmission lines are maintained... Above ground electric transmission lines are NOT maintained.... Recommended Mitigation Strategies.. ◻.; Know who to call should there be a problem with electric lines in community...... Topographical Features.. No topographical features adversely affect wildland fire behavior... → Topographical features adversely affect wildland fire behavior (box canyons, chimneys, etc.).... Recommended Mitigation Strategies.. ◻.; Maintain situational awareness of fire danger in your area, as local topographical features can adversely affect wildland behavior...... |
| --- | --- |

|  | Adjacency to Wildlands.. Not adjacent to wildlands with accumulated fuels... → Adjacent to wildlands with accumulated fuels.... Recommended Mitigation Strategies.. ◻.; When possible, install firebreaks and reduce fuel loads around community boundary to reduce risk from adjacent wildlands; Work with neighboring land owners...... Undeveloped Lots with Restricted Access and/or Not Maintained.. Fewer than 10% of lots are undeveloped... → 10% to 30% of lots are undeveloped... 31% to 50% of lots are undeveloped... Greater than 51% of lots are undeveloped.... Recommended Mitigation Strategies.. ◻.; Provide Living with Fire/Firewise construction guidelines to developers /owners... ◻.; Consider developing covenant restrictions, if applicable...... |
| --- | --- |

STRUCTURES ASSESSMENT.. Roofing Materials.. → > 75% of homes have metal, tile or class A asphalt or fiberglass shingles... 50 to 75% of homes have metal, tile or class A asphalt or fiberglass shingles... < 50% of homes have metal, tile or class A asphalt or fiberglass shingles.... Recommended Mitigation Strategies.. ◻.; Use fire-resistant roofing material such as metal, tile, or Class A shingles... ◻.; Inspect for and address gaps in roofing that can expose roof decking or supports... ◻.; Place angle flashing over openings between the roof decking and fascia board...... Debris on Roof and/or Gutters.. → No... Yes.... Recommended Mitigation Strategies.. ◻.; Clear branch, leaf-litter and other debris from roof and gutters regularly... ◻.; Prune tree limbs away from roof...... Ventilation and Soffits.. → > 75% of homes have non-combustible ventilation soffits with mesh or screening... 50-74% of homes have non-combustible ventilation soffits with mesh or screening... < 50% of homes have non-combustible ventilation soffits with mesh or screening.... Recommended Mitigation Strategies.. ◻.; Clean vents to keep them free of debris, allowing them to keep embers out while allowing air flow for ventilation... ◻.; Enclose or box-in eaves with non-combustible materials such as metal, cement board or stucco... ◻.; Install a 1/8 inch metal screen behind roof vents...... Siding.. → > 75% of homes have non-combustible siding... 50-74% of homes have non-combustible siding... < 50% of homes have non-combustible siding.... Recommended Mitigation Strategies.. ◻.; Keep landscaping materials and vegetation away from combustible siding... ◻.; Create 5-foot non-combustible area (Zone 0) around house... ◻.; Replace with noncombustible siding when possible......

Underskirting.. > 75% of homes have skirting underneath raised floors/decks... 50-74% of homes have skirting underneath... → < 50% of homes have skirting underneath.... Recommended Mitigation Strategies.. ◻.; Remove combustible vegetation and leaf litter... ◻.; Spread gravel or other non-combustible material under the deck... ◻.; Screen in the bottom of the deck with metal 1/8-inch screening... ◻.; Separate wooden fences from the house with a stone or metal barrier...... Wooden Attachments.. → > 75% of homes have NO Wooden Attachments... 50-74% of homes have NO Wooden Attachments... < 50% of homes have NO Wooden Attachments.... Recommended Mitigation Strategies.. ◻.; Maintain debris-free decks (e.g., remove ignitable furniture, planters and covering propane grills, especially during high fire danger periods)... ◻.; Consider disconnecting fences from structures, or replacing materials directly attached to structures with fire resistant materials... ◻.; Be aware that wooden attachments can act as a fuse to the structure...... Building Setback.. → Not applicable... Greater than or equal to 30 feet from slope... Less than 30 feet from slope.... Recommended Mitigation Strategies.. ◻.; N/A...... Propane.. > 30 feet from the house and surrounding vegetation maintained... Fewer than 30 feet from the house and/or surrounding vegetation not maintained... → N/A.... Recommended Mitigation Strategies.. ◻.; N/A......

| Electric Utilities.. Electric Underground... → Electric Overhead drop maintained... Electric Overhead drop not maintained.... Recommended Mitigation Strategies.. ◻.; Â Keep vegetation pruned and mowed around electric cabinets... ◻.; Place non-flammable mulch (rock, stone) around base of electrical cabinets... ◻.; Plant less flammable bushes and shrubs around electrical cabinets...... Non-Combustible Zone 0.. → > 75% of homes/outbuildings have adjacent 5-ft non-combustible zone... 50-74% of homes/outbuildings have adjacent 5-ft non-combustible zone... < 50% of homes/outbuildings have adjacent 5-ft non-combustible zone.... Recommended Mitigation Strategies.. ◻.; Remove flammable materials and Create 5-foot non-combustible area (Zone 0) around house...... |
| --- |
|  |  | COMMENTS. |  |

## Dry Lake

|  |  | Community Wildfire Risk Assessment.. Total Assessed Rating.. 143 - High.. Suppression Rating.. Extreme Hazard.. Surrounding Environment Rating.. High Hazard.. Structures Rating.. Moderate Hazard.. Fire Protection District.. Carson City Fire.. Fireshed(s).. Carson Valley-Carson River.. Community Information.. Latitude.; 39° 8' 12".. Longitude.; -119° 48' 29".. Dwelling Units.; 4.. Size.; 4.85 acres.. Community Type.; Residential - Stick-Built.. Assessed By:.; Scott OBrien.. Assessment Date:.; 03-27-2023.. SUPPRESSION ASSESSMENT.. Ingress and Egress.. 2 or more roads in/out with NO response/evacuation complexity... 2 or more roads in/out with SLIGHT response/evacuation complexity... 2 or more roads in/out with MODERATE/HIGH response/evacuation complexity... → One road in and out (entrance and exit is the same).... Recommended Mitigation Strategies.. ◻.; Keep community ingress/egress open and maintained (cleared of vegetation, vehicles, and/or any obstructions)... ◻.; Consider developing safety zones and a plan for Shelter-in-Place; consult with Local Fire Department... ◻.; Consider adding a secondary ingress / egress route for use in emergencies...... Road Width.. Road width is > 24 feet... Road width is > 20 feet and < 24 feet... → Road width is < 20 feet.... Recommended Mitigation Strategies.. ◻.; Keep shoulders of road clear for emergency vehicle use at all times... ◻.; Consider providing pull-offs every 100 yards for emergency vehicle use... ◻.; Coordinate with fire department to ensure they are aware of road width limitations... ◻.; Be aware that road width could limit emergency vehicles to brush trucks only...... Road Accessibility.. Surfaced road... → Non-surfaced road, grade less than or equal to 5%... Non-surfaced road, grade greater than 5%... Non-maintained dirt road.... Recommended Mitigation Strategies.. ◻.; Consider road improvements to reduce the risk driving in extreme grades, tight corners, and road intersections... ◻.; Coordinate with fire department to test access with emergency response vehicles....... Secondary Road Terminus.. Roads ends in a cul-de-sac, diameter > 100 feet... Roads ends in a cul-de-sac, diameter < 100 feet... Dead end roads <200 feet long... → Dead end roads >200 feet long.... Recommended Mitigation Strategies.. ◻.; Ensure emergency responders are aware of dead-end roads; Consider signing all dead ends.... ◻.; If dead-end roads are narrow, restrict access during an emergency...... Street Signs.. Present throughout, lettering 4 inches high, non-flammable and reflective... Inconsistent throughout, lettering 4 inches high, non-flammable and reflective... → Present or inconsistent but wooden, non-reflective, or lettering less than 4"... Not present.... Recommended Mitigation Strategies.. ◻.; Consider upgrading to reflective, noncombustible street signs to improve emergency response efforts... ◻.; Keep street signs visible and clear of vegetation and fine fuels...... Driveways.. Average driveway allows access to homes... → Average driveway restricts access to homes.... Recommended Mitigation Strategies.. ◻.; Improve driveway accessibility where possible... ◻.; Ensure emergency responders are aware of driveway restrictions...... Water Supply.. Pressurized hydrants spaced less than 1000 feet apart... Pressurized hydrants spaced more than 1000 feet apart... Dry Hydrant(s) / Draft available within the community... → Other accessible sources within community (pond, lake, etc.)... Water sources located within 4 miles of community (incl heli dip sites)... No water sources in or within 4 miles of the community.... Recommended Mitigation Strategies.. ◻.; Coordinate with fire department and land owners to train/test use of local water sources (e.g. ponds, lakes)....... Geographic Features.. No notable geographical features present to hinder fire suppression... → Suppression efforts hindered by geographical features (e.g., hazardous terrain).... Recommended Mitigation Strategies.. ◻.; Ensure emergency responders are aware of local geographic features that can hinder fire suppression efforts; consider pre-suppression plan...... Local Response Resources.. 5 mi. or less from Agency with Response Authority (Staffed FD)... 5 mi. or less from Agency with Response Authority (Mixed Staff/VFD)... 5 mi. or less from Agency with Response Authority (VFD)... → > 5 mi. from Agency with Response Authority FD.... Recommended Mitigation Strategies.. ◻.; Establish and maintain contact with the closest Fire Department; consider pre-suppression plan... ◻.; Discuss with closest Fire Department to identify quicker response strategies and other potential solutions...... Community Organization/Governance.. GID present; HAS structure for sustained fire prevention and mitigation... HOA present; HAS structure for sustained fire prevention and mitigation... Municipal govt present; HAS structure for sustained fire prevention and mitigation... GID present; LACKS structure for sustained fire prevention and mitigation... HOA present; LACKS structure for sustained fire prevention and mitigation... Municipal govt present; LACKS structure for sustained fire prevention and mitigation... → Lacks any structure for sustained fire prevention and mitigation.... Recommended Mitigation Strategies.. ◻.; Work with community to become more proactive towards protecting your life and property against wildfires; Become a Firewise USA® Site... ◻.; Host a Community Education Event at least once a year... ◻.; Complete Community Risk Mitigation Project(s) as identified by Community Action Plan... ◻.; Ensure individual homes are ignition-resistant, hardened, and Firewise/Living with Fire concepts are followed........ SURROUNDING ENVIRONMENT ASSESSMENT.. Predominant Vegetation.. Light (grass)... → Medium (brush)... Heavy (timber, overgrown sage, Pinyon/Juniper with dead/down, etc.)... Extreme / Slash (Any Combination of contiguous Light, Medium, Heavy).... Recommended Mitigation Strategies.. ◻.; Consider removal of ladder fuels that allow fire to climb from lower to higher vegetation... ◻.; Trim tree canopies regularly to keep their branches a minimum of 10' from structures and other trees... ◻.; Leave 30 feet between clusters of two to three trees, or 20 feet between individual trees... ◻.; Prune trees 6-10 feet from the ground...... Defensible Space.. > 75% of homes meet criteria in Zone 0, 1 & 2... 50 to 75% of homes meet criteria in Zone 0, 1 & 2... → < 50% of homes meet criteria in Zone 0, 1 & 2 - Light fuels amongst structures... < 50% of homes meet criteria in Zone 0, 1 & 2 - Moderate fuels amongst structures... Fuels heavy/extreme amongst structures & other urban hazards/materials are present.... Recommended Mitigation Strategies.. ◻.; N/A....... Structure-to-Structure Ignition.. → No Possible Structure-to-Structure Ignition... Possible Structure-to-Structure Ignition.... Recommended Mitigation Strategies.. ◻.; N/A...... Slope.. → Slope 0% - 5%... Slope 6 % - 10%... Slope 11% - 30%... Slope > 31%.... Recommended Mitigation Strategies.. ◻.; N/A...... Vegetation on Electric Transmission Lines.. No above ground electric transmission lines present... → Above ground electric transmission lines are maintained... Above ground electric transmission lines are NOT maintained.... Recommended Mitigation Strategies.. ◻.; Know who to call should there be a problem with electric lines in community...... Topographical Features.. No topographical features adversely affect wildland fire behavior... → Topographical features adversely affect wildland fire behavior (box canyons, chimneys, etc.).... Recommended Mitigation Strategies.. ◻.; Maintain situational awareness of fire danger in your area, as local topographical features can adversely affect wildland behavior....... Adjacency to Wildlands.. Not adjacent to wildlands with accumulated fuels... → Adjacent to wildlands with accumulated fuels.... Recommended Mitigation Strategies.. ◻.; When possible, install firebreaks and reduce fuel loads around community boundary to reduce risk from adjacent wildlands; Work with neighboring land owners...... Undeveloped Lots with Restricted Access and/or Not Maintained.. Fewer than 10% of lots are undeveloped... 10% to 30% of lots are undeveloped... 31% to 50% of lots are undeveloped... → Greater than 51% of lots are undeveloped.... Recommended Mitigation Strategies.. ◻.; Provide Living with Fire/Firewise construction guidelines to developers /owners... ◻.; Consider developing covenant restrictions, if applicable....... STRUCTURES ASSESSMENT.. Roofing Materials.. → > 75% of homes have metal, tile or class A asphalt or fiberglass shingles... 50 to 75% of homes have metal, tile or class A asphalt or fiberglass shingles... < 50% of homes have metal, tile or class A asphalt or fiberglass shingles.... Recommended Mitigation Strategies.. ◻.; Use fire-resistant roofing material such as metal, tile, or Class A shingles... ◻.; Inspect for and address gaps in roofing that can expose roof decking or supports... ◻.; Place angle flashing over openings between the roof decking and fascia board...... Debris on Roof and/or Gutters.. → No... Yes.... Recommended Mitigation Strategies.. ◻.; Clear branch, leaf-litter and other debris from roof and gutters regularly... ◻.; Prune tree limbs away from roof...... Ventilation and Soffits.. → > 75% of homes have non-combustible ventilation soffits with mesh or screening... 50-74% of homes have non-combustible ventilation soffits with mesh or screening... < 50% of homes have non-combustible ventilation soffits with mesh or screening.... Recommended Mitigation Strategies.. ◻.; Clean vents to keep them free of debris, allowing them to keep embers out while allowing air flow for ventilation... ◻.; Enclose or box-in eaves with non-combustible materials such as metal, cement board or stucco... ◻.; Install a 1/8 inch metal screen behind roof vents...... Siding.. > 75% of homes have non-combustible siding... → 50-74% of homes have non-combustible siding... < 50% of homes have non-combustible siding.... Recommended Mitigation Strategies.. ◻.; Keep landscaping materials and vegetation away from combustible siding... ◻.; Create 5-foot non-combustible area (Zone 0) around house... ◻.; Replace with noncombustible siding when possible....... Underskirting.. > 75% of homes have skirting underneath raised floors/decks... → 50-74% of homes have skirting underneath... < 50% of homes have skirting underneath.... Recommended Mitigation Strategies.. ◻.; Remove combustible vegetation and leaf litter... ◻.; Spread gravel or other non-combustible material under the deck... ◻.; Screen in the bottom of the deck with metal 1/8-inch screening... ◻.; Separate wooden fences from the house with a stone or metal barrier...... Wooden Attachments.. > 75% of homes have NO Wooden Attachments... → 50-74% of homes have NO Wooden Attachments... < 50% of homes have NO Wooden Attachments.... Recommended Mitigation Strategies.. ◻.; Â Maintain debris-free decks (e.g., remove ignitable furniture, planters and covering propane grills, especially during high fire danger periods)... ◻.; Consider disconnecting fences from structures, or replacing materials directly attached to structures with fire resistant materials... ◻.; Be aware that wooden attachments can act as a fuse to the structure...... Building Setback.. → Not applicable... Greater than or equal to 30 feet from slope... Less than 30 feet from slope.... Recommended Mitigation Strategies.. ◻.; N/A...... Propane.. → > 30 feet from the house and surrounding vegetation maintained... Fewer than 30 feet from the house and/or surrounding vegetation not maintained... N/A.... Recommended Mitigation Strategies.. ◻.; N/A....... Electric Utilities.. Electric Underground... → Electric Overhead drop maintained... Electric Overhead drop not maintained.... Recommended Mitigation Strategies.. ◻.; Â Keep vegetation pruned and mowed around electric cabinets... ◻.; Place non-flammable mulch (rock, stone) around base of electrical cabinets... ◻.; Plant less flammable bushes and shrubs around electrical cabinets...... Non-Combustible Zone 0.. > 75% of homes/outbuildings have adjacent 5-ft non-combustible zone... → 50-74% of homes/outbuildings have adjacent 5-ft non-combustible zone... < 50% of homes/outbuildings have adjacent 5-ft non-combustible zone.... Recommended Mitigation Strategies.. ◻.; N/A....... COMMENTS.. |  |
| --- | --- | --- | --- |

## North Curry Street

|  |  | Community Wildfire Risk Assessment. |  |
| --- | --- | --- | --- |
|  |  | Total Assessed Rating. |  |  |  |  |  |
|  |  | 102 - High. |  |  |  |  |  |
|  |  | Suppression Rating. |  |  |  |  |
|  |  | Moderate Hazard. |  |  |  |
|  |  | Surrounding Environment Rating. |  |  |  |  |
|  |  | High Hazard. |  |  |  |
|  |  | Structures Rating. |  |  |  |  |
|  |  | Moderate Hazard. |  |  |  |
|  |  | Fire Protection District. |  |  |  |  |
|  |  | Carson City Fire. |  |  |  |
|  |  | Fireshed(s). |  |  |  |  |
|  | Carson Valley-Carson River. |  |  |  |
|  | Community Information. |  |  |  |  |
|  |  | Latitude. | 39° 8' 40". |  |  |  |
|  |  | Longitude. | -119° 46' 14". |  |  |  |
|  |  | Dwelling Units. | 50. |  |  |  |
|  |  | Size. | 106.66 acres. |  |  |  |
|  |  | Community Type. | Residential/Commercial. |  |  |  |
|  | Assessed By:. | Scott OBrien. |  |  |  |
|  |  | Assessment Date:. | 03-27-2023. |  |  |  |

|  | SUPPRESSION ASSESSMENT.. Ingress and Egress.. → 2 or more roads in/out with NO response/evacuation complexity... 2 or more roads in/out with SLIGHT response/evacuation complexity... 2 or more roads in/out with MODERATE/HIGH response/evacuation complexity... One road in and out (entrance and exit is the same).... Recommended Mitigation Strategies.. ◻.; Keep community ingress/egress open and maintained (cleared of vegetation)... ◻.; Develop community plan for evacuation routes, safe zones, staging areas... ◻.; If community is gated, develop evacuation plan, and ensure emergency responder access... ◻.; Ensure residents know their closest exit in case of emergency... ◻.; Ask Local Fire Department about Ready, Set, Go!...... Road Width.. Road width is > 24 feet... → Road width is > 20 feet and < 24 feet... Road width is < 20 feet.... Recommended Mitigation Strategies.. ◻.; Keep shoulders of road clear for emergency vehicle use at all times... ◻.; Consider providing pull-offs every 100 yards...... Road Accessibility.. → Surfaced road... Non-surfaced road, grade less than or equal to 5%... Non-surfaced road, grade greater than 5%... Non-maintained dirt road.... Recommended Mitigation Strategies.. ◻.; Ensure that road maintenance plan is in place...... |
| --- | --- |

|  | Secondary Road Terminus.. Roads ends in a cul-de-sac, diameter > 100 feet... Roads ends in a cul-de-sac, diameter < 100 feet... → Dead end roads <200 feet long... Dead end roads >200 feet long.... Recommended Mitigation Strategies.. ◻.; Ensure emergency responder are aware of dead-end roads; Consider signing all dead ends....... Street Signs.. → Present throughout, lettering 4 inches high, non-flammable and reflective... Inconsistent throughout, lettering 4 inches high, non-flammable and reflective... Present or inconsistent but wooden, non-reflective, or lettering less than 4"... Not present.... Recommended Mitigation Strategies.. ◻.; Keep street signs visible and clear of vegetation and fine fuels...... Driveways.. → Average driveway allows access to homes... Average driveway restricts access to homes.... Recommended Mitigation Strategies.. ◻.; Maintain driveway access and clearance...... Water Supply.. → Pressurized hydrants spaced less than 1000 feet apart... Pressurized hydrants spaced more than 1000 feet apart... Dry Hydrant(s) / Draft available within the community... Other accessible sources within community (pond, lake, etc.)... Water sources located within 4 miles of community (incl heli dip sites)... No water sources in or within 4 miles of the community.... Recommended Mitigation Strategies.. ◻.; Ensure hydrants and water sources are marked, accessible and properly maintained... ◻.; Keep hydrants clear of obstructions and vegetation...... |
| --- | --- |

|  | Geographic Features.. No notable geographical features present to hinder fire suppression... → Suppression efforts hindered by geographical features (e.g., hazardous terrain).... Recommended Mitigation Strategies.. ◻.; Ensure emergency responders are aware of local geographic features that can hinder fire suppression efforts; consider pre-suppression plan...... Local Response Resources.. → 5 mi. or less from Agency with Response Authority (Staffed FD)... 5 mi. or less from Agency with Response Authority (Mixed Staff/VFD)... 5 mi. or less from Agency with Response Authority (VFD)... > 5 mi. from Agency with Response Authority FD.... Recommended Mitigation Strategies.. ◻.; Establish and maintain contact with the closest Fire Department; consider pre-suppression plan... ◻.; Be aware of the importance of early detection and reporting of any emergency...... Community Organization/Governance.. GID present; HAS structure for sustained fire prevention and mitigation... HOA present; HAS structure for sustained fire prevention and mitigation... → Municipal govt present; HAS structure for sustained fire prevention and mitigation... GID present; LACKS structure for sustained fire prevention and mitigation... HOA present; LACKS structure for sustained fire prevention and mitigation... Municipal govt present; LACKS structure for sustained fire prevention and mitigation... Lacks any structure for sustained fire prevention and mitigation.... Recommended Mitigation Strategies.. ◻.; Host a Community Education Event at least once a year; Become a Firewise USA® Site... ◻.; Complete Community Risk Mitigation Project(s) as identified by Community Action Plan...... |
| --- | --- |

|  | SURROUNDING ENVIRONMENT ASSESSMENT.. Predominant Vegetation.. Light (grass)... → Medium (brush)... Heavy (timber, overgrown sage, Pinyon/Juniper with dead/down, etc.)... Extreme / Slash (Any Combination of contiguous Light, Medium, Heavy).... Recommended Mitigation Strategies.. ◻.; Consider removal of ladder fuels that allow fire to climb from lower to higher vegetation... ◻.; Trim tree canopies regularly to keep their branches a minimum of 10' from structures and other trees... ◻.; Leave 30 feet between clusters of two to three trees, or 20 feet between individual trees... ◻.; Prune trees 6-10 feet from the ground...... Defensible Space.. > 75% of homes meet criteria in Zone 0, 1 & 2... → 50 to 75% of homes meet criteria in Zone 0, 1 & 2... < 50% of homes meet criteria in Zone 0, 1 & 2 - Light fuels amongst structures... < 50% of homes meet criteria in Zone 0, 1 & 2 - Moderate fuels amongst structures... Fuels heavy/extreme amongst structures & other urban hazards/materials are present.... Recommended Mitigation Strategies.. ◻.; Be aware of the risks from falling embers in relation to nearby fuels and defensible space... ◻.; Mow lawns regularly... ◻.; Water grass, plants, trees, and mulch regularly... ◻.; Create a spacing of 30 feet between tree crowns... ◻.; Create a non-combustible area (zone 0) within 5 feet of your home, using non-flammable landscaping materials... ◻.; Remove dead vegetation from under the deck and within 10 feet of the house; stack firewood away from structures... ◻.; Consider xeriscaping... ◻.; Plant a mixture of deciduous trees (e.g., oak and maple) and coniferous trees (e.g. pine)... ◻.; Create fuel breaks like driveways and gravel walkways...... |
| --- | --- |

|  | Structure-to-Structure Ignition.. No Possible Structure-to-Structure Ignition... → Possible Structure-to-Structure Ignition.... Recommended Mitigation Strategies.. ◻.; Work with neighbors to remove/prune vegetation between houses to mitigate structure-to-structure ignition risk; consider non-combustible fencing 5 feet from structure... ◻.; Consider use of sprinkler systems to keep vegetation moisture levels up... ◻.; Replace flammable roofs, siding, soffits, etc. with nonflammable when possible...... Slope.. → Slope 0% - 5%... Slope 6 % - 10%... Slope 11% - 30%... Slope > 31%.... Recommended Mitigation Strategies.. ◻.; N/A...... Vegetation on Electric Transmission Lines.. No above ground electric transmission lines present... → Above ground electric transmission lines are maintained... Above ground electric transmission lines are NOT maintained.... Recommended Mitigation Strategies.. ◻.; Know who to call should there be a problem with electric lines in community...... Topographical Features.. No topographical features adversely affect wildland fire behavior... → Topographical features adversely affect wildland fire behavior (box canyons, chimneys, etc.).... Recommended Mitigation Strategies.. ◻.; Maintain situational awareness of fire danger in your area, as local topographical features can adversely affect wildland behavior...... |
| --- | --- |

|  | Adjacency to Wildlands.. Not adjacent to wildlands with accumulated fuels... → Adjacent to wildlands with accumulated fuels.... Recommended Mitigation Strategies.. ◻.; When possible, install firebreaks and reduce fuel loads around community boundary to reduce risk from adjacent wildlands; Work with neighboring land owners...... Undeveloped Lots with Restricted Access and/or Not Maintained.. Fewer than 10% of lots are undeveloped... → 10% to 30% of lots are undeveloped... 31% to 50% of lots are undeveloped... Greater than 51% of lots are undeveloped.... Recommended Mitigation Strategies.. ◻.; Provide Living with Fire/Firewise construction guidelines to developers /owners... ◻.; Consider developing covenant restrictions, if applicable...... |
| --- | --- |

STRUCTURES ASSESSMENT.. Roofing Materials.. → > 75% of homes have metal, tile or class A asphalt or fiberglass shingles... 50 to 75% of homes have metal, tile or class A asphalt or fiberglass shingles... < 50% of homes have metal, tile or class A asphalt or fiberglass shingles.... Recommended Mitigation Strategies.. ◻.; Use fire-resistant roofing material such as metal, tile, or Class A shingles... ◻.; Inspect for and address gaps in roofing that can expose roof decking or supports... ◻.; Place angle flashing over openings between the roof decking and fascia board...... Debris on Roof and/or Gutters.. → No... Yes.... Recommended Mitigation Strategies.. ◻.; Clear branch, leaf-litter and other debris from roof and gutters regularly... ◻.; Prune tree limbs away from roof...... Ventilation and Soffits.. → > 75% of homes have non-combustible ventilation soffits with mesh or screening... 50-74% of homes have non-combustible ventilation soffits with mesh or screening... < 50% of homes have non-combustible ventilation soffits with mesh or screening.... Recommended Mitigation Strategies.. ◻.; Clean vents to keep them free of debris, allowing them to keep embers out while allowing air flow for ventilation... ◻.; Enclose or box-in eaves with non-combustible materials such as metal, cement board or stucco... ◻.; Install a 1/8 inch metal screen behind roof vents...... Siding.. > 75% of homes have non-combustible siding... → 50-74% of homes have non-combustible siding... < 50% of homes have non-combustible siding.... Recommended Mitigation Strategies.. ◻.; Keep landscaping materials and vegetation away from combustible siding... ◻.; Create 5-foot non-combustible area (Zone 0) around house... ◻.; Replace with noncombustible siding when possible......

Underskirting.. > 75% of homes have skirting underneath raised floors/decks... 50-74% of homes have skirting underneath... → < 50% of homes have skirting underneath.... Recommended Mitigation Strategies.. ◻.; Remove combustible vegetation and leaf litter... ◻.; Spread gravel or other non-combustible material under the deck... ◻.; Screen in the bottom of the deck with metal 1/8-inch screening... ◻.; Separate wooden fences from the house with a stone or metal barrier...... Wooden Attachments.. > 75% of homes have NO Wooden Attachments... → 50-74% of homes have NO Wooden Attachments... < 50% of homes have NO Wooden Attachments.... Recommended Mitigation Strategies.. ◻.; Â Maintain debris-free decks (e.g. remove ignitable furniture, planters and covering propane grills, especially during high fire danger periods)... ◻.; Consider disconnecting fences from structures, or replacing materials directly attached to structures with fire resistant materials... ◻.; Be aware that wooden attachments can act as a fuse to the structure...... Building Setback.. → Not applicable... Greater than or equal to 30 feet from slope... Less than 30 feet from slope.... Recommended Mitigation Strategies.. ◻.; N/A...... Propane.. > 30 feet from the house and surrounding vegetation maintained... Fewer than 30 feet from the house and/or surrounding vegetation not maintained... → N/A.... Recommended Mitigation Strategies.. ◻.; N/A......

| Electric Utilities.. Electric Underground... → Electric Overhead drop maintained... Electric Overhead drop not maintained.... Recommended Mitigation Strategies.. ◻.; Â Keep vegetation pruned and mowed around electric cabinets... ◻.; Place non-flammable mulch (rock, stone) around base of electrical cabinets... ◻.; Plant less flammable bushes and shrubs around electrical cabinets...... Non-Combustible Zone 0.. > 75% of homes/outbuildings have adjacent 5-ft non-combustible zone... → 50-74% of homes/outbuildings have adjacent 5-ft non-combustible zone... < 50% of homes/outbuildings have adjacent 5-ft non-combustible zone.... Recommended Mitigation Strategies.. ◻.; N/A...... |
| --- |
|  |  | COMMENTS. |  |

## South Curry Street

|  |  | Community Wildfire Risk Assessment. |  |
| --- | --- | --- | --- |
|  |  | Total Assessed Rating. |  |  |  |  |  |
|  |  | 95 - Moderate. |  |  |  |  |  |
|  |  | Suppression Rating. |  |  |  |  |
|  |  | Low Hazard. |  |  |  |
|  |  | Surrounding Environment Rating. |  |  |  |  |
|  |  | High Hazard. |  |  |  |
|  |  | Structures Rating. |  |  |  |  |
|  |  | Moderate Hazard. |  |  |  |
|  |  | Fire Protection District. |  |  |  |  |
|  |  | Carson City Fire. |  |  |  |
|  |  | Fireshed(s). |  |  |  |  |
|  | Carson Valley-Carson River. |  |  |  |
|  | Community Information. |  |  |  |  |
|  |  | Latitude. | 39° 7' 59". |  |  |  |
|  |  | Longitude. | -119° 46' 23". |  |  |  |
|  |  | Dwelling Units. | 20. |  |  |  |
|  |  | Size. | 76.62 acres. |  |  |  |
|  |  | Community Type. | Residential/Commercial. |  |  |  |
|  | Assessed By:. | Scott OBrien. |  |  |  |
|  |  | Assessment Date:. | 03-27-2023. |  |  |  |

|  | SUPPRESSION ASSESSMENT.. Ingress and Egress.. → 2 or more roads in/out with NO response/evacuation complexity... 2 or more roads in/out with SLIGHT response/evacuation complexity... 2 or more roads in/out with MODERATE/HIGH response/evacuation complexity... One road in and out (entrance and exit is the same).... Recommended Mitigation Strategies.. ◻.; Keep community ingress/egress open and maintained (cleared of vegetation)... ◻.; Develop community plan for evacuation routes, safe zones, staging areas... ◻.; If community is gated, develop evacuation plan and ensure emergency responder access... ◻.; Ensure residents know their closest exit in case of emergency... ◻.; Ask Local Fire Department about Ready, Set, Go!...... Road Width.. Road width is > 24 feet... → Road width is > 20 feet and < 24 feet... Road width is < 20 feet.... Recommended Mitigation Strategies.. ◻.; Keep shoulders of road clear for emergency vehicle use at all times... ◻.; Consider providing pull-offs every 100 yards...... Road Accessibility.. → Surfaced road... Non-surfaced road, grade less than or equal to 5%... Non-surfaced road, grade greater than 5%... Non-maintained dirt road.... Recommended Mitigation Strategies.. ◻.; Ensure that road maintenance plan is in place...... |
| --- | --- |

|  | Secondary Road Terminus.. → Roads ends in a cul-de-sac, diameter > 100 feet... Roads ends in a cul-de-sac, diameter < 100 feet... Dead end roads <200 feet long... Dead end roads >200 feet long.... Recommended Mitigation Strategies.. ◻.; Maintain unobstructed access into cul-de-sacs... ◻.; Ensure cul-de-sacs are free of vehicles and/or other items...... Street Signs.. → Present throughout, lettering 4 inches high, non-flammable and reflective... Inconsistent throughout, lettering 4 inches high, non-flammable and reflective... Present or inconsistent but wooden, non-reflective, or lettering less than 4"... Not present.... Recommended Mitigation Strategies.. ◻.; Keep street signs visible and clear of vegetation and fine fuels...... Driveways.. → Average driveway allows access to homes... Average driveway restricts access to homes.... Recommended Mitigation Strategies.. ◻.; Maintain driveway access and clearance...... Water Supply.. → Pressurized hydrants spaced less than 1000 feet apart... Pressurized hydrants spaced more than 1000 feet apart... Dry Hydrant(s) / Draft available within the community... Other accessible sources within community (pond, lake, etc.)... Water sources located within 4 miles of community (incl heli dip sites)... No water sources in or within 4 miles of the community.... Recommended Mitigation Strategies.. ◻.; Ensure hydrants and water sources are marked, accessible and properly maintained... ◻.; Keep hydrants clear of obstructions and vegetation...... |
| --- | --- |

|  | Geographic Features.. No notable geographical features present to hinder fire suppression... → Suppression efforts hindered by geographical features (e.g. hazardous terrain).... Recommended Mitigation Strategies.. ◻.; Ensure emergency responders are aware of local geographic features that can hinder fire suppression efforts; consider pre-suppression plan...... Local Response Resources.. → 5 mi. or less from Agency with Response Authority (Staffed FD)... 5 mi. or less from Agency with Response Authority (Mixed Staff/VFD)... 5 mi. or less from Agency with Response Authority (VFD)... > 5 mi. from Agency with Response Authority FD.... Recommended Mitigation Strategies.. ◻.; Establish and maintain contact with the closest Fire Department; consider pre-suppression plan... ◻.; Be aware of the importance of early detection and reporting of any emergency...... Community Organization/Governance.. GID present; HAS structure for sustained fire prevention and mitigation... HOA present; HAS structure for sustained fire prevention and mitigation... → Municipal govt present; HAS structure for sustained fire prevention and mitigation... GID present; LACKS structure for sustained fire prevention and mitigation... HOA present; LACKS structure for sustained fire prevention and mitigation... Municipal govt present; LACKS structure for sustained fire prevention and mitigation... Lacks any structure for sustained fire prevention and mitigation.... Recommended Mitigation Strategies.. ◻.; Host a Community Education Event at least once a year; Become a Firewise USA® Site... ◻.; Complete Community Risk Mitigation Project(s) as identified by Community Action Plan...... |
| --- | --- |

|  | SURROUNDING ENVIRONMENT ASSESSMENT.. Predominant Vegetation.. Light (grass)... → Medium (brush)... Heavy (timber, overgrown sage, Pinyon/Juniper with dead/down, etc)... Extreme / Slash (Any Combination of contiguous Light, Medium, Heavy).... Recommended Mitigation Strategies.. ◻.; Consider removal of ladder fuels that allow fire to climb from lower to higher vegetation... ◻.; Trim tree canopies regularly to keep their branches a minimum of 10' from structures and other trees... ◻.; Leave 30 feet between clusters of two to three trees, or 20 feet between individual trees... ◻.; Prune trees 6-10 feet from the ground...... Defensible Space.. > 75% of homes meet criteria in Zone 0, 1 & 2... → 50 to 75% of homes meet criteria in Zone 0, 1 & 2... < 50% of homes meet criteria in Zone 0, 1 & 2 - Light fuels amongst structures... < 50% of homes meet criteria in Zone 0, 1 & 2 - Moderate fuels amongst structures... Fuels heavy/extreme amongst structures & other urban hazards/materials are present.... Recommended Mitigation Strategies.. ◻.; Be aware of the risks from falling embers in relation to nearby fuels and defensible space... ◻.; Mow lawns regularly... ◻.; Water grass, plants, trees and mulch regularly... ◻.; Create a spacing of 30 feet between tree crowns... ◻.; Create a non-combustible area (zone 0) within 5 feet of your home, using non-flammable landscaping materials... ◻.; Remove dead vegetation from under the deck and within 10 feet of the house; stack firewood away from structures... ◻.; ConsiderÂ xeriscaping... ◻.; Plant a mixture of deciduous trees (e.g. oak and maple) and coniferous trees (e.g. pine)... ◻.; Create fuel breaks like driveways and gravel walkways...... |
| --- | --- |

|  | Structure-to-Structure Ignition.. No Possible Structure-to-Structure Ignition... → Possible Structure-to-Structure Ignition.... Recommended Mitigation Strategies.. ◻.; Work with neighbors to remove/prune vegetation between houses to mitigate structure-to-structure ignition risk; consder non-combustible fencing 5 feet from structure... ◻.; Consider use of sprinkler systems to keep vegetation moisture levels up... ◻.; Replace flammable roofs, siding, soffits, etc. with nonflammable when possible...... Slope.. Slope 0% - 5%... → Slope 6 % - 10%... Slope 11% - 30%... Slope > 31%.... Recommended Mitigation Strategies.. ◻.; N/A...... Vegetation on Electric Transmission Lines.. No above ground electric transmission lines present... → Above ground electric transmission lines are maintained... Above ground electric transmission lines are NOT maintained.... Recommended Mitigation Strategies.. ◻.; Know who to call should there be a problem with electric lines in community...... Topographical Features.. No topographical features adversely affect wildland fire behavior... → Topographical features adversely affect wildland fire behavior (box canyons, chimneys, etc.).... Recommended Mitigation Strategies.. ◻.; Maintain situational awareness of fire danger in your area, as local topographical features can adversely affect wildland behavior...... |
| --- | --- |

|  | Adjacency to Wildlands.. Not adjacent to wildlands with accumulated fuels... → Adjacent to wildlands with accumulated fuels.... Recommended Mitigation Strategies.. ◻.; When possible, install firebreaks and reduce fuel loads around community boundary to reduce risk from adjacent wildlands; Work with neighboring land owners...... Undeveloped Lots with Restricted Access and/or Not Maintained.. Fewer than 10% of lots are undeveloped... → 10% to 30% of lots are undeveloped... 31% to 50% of lots are undeveloped... Greater than 51% of lots are undeveloped.... Recommended Mitigation Strategies.. ◻.; Provide Living with Fire/Firewise construction guidelines to developers /owners... ◻.; Consider developing covenant restrictions, if applicable...... |
| --- | --- |

STRUCTURES ASSESSMENT.. Roofing Materials.. → > 75% of homes have metal, tile or class A asphalt or fiberglass shingles... 50 to 75% of homes have metal, tile or class A asphalt or fiberglass shingles... < 50% of homes have metal, tile or class A asphalt or fiberglass shingles.... Recommended Mitigation Strategies.. ◻.; Use fire-resistant roofing material such as metal, tile or Class A shingles... ◻.; Inspect for and address gaps in roofing that can expose roof decking or supports... ◻.; Place angle flashing over openings between the roof decking and fascia board...... Debris on Roof and/or Gutters.. → No... Yes.... Recommended Mitigation Strategies.. ◻.; Clear branch, leaf-litter and other debris from roof and gutters regularly... ◻.; Prune tree limbs away from roof...... Ventilation and Soffits.. → > 75% of homes have non-combustible ventilation soffits with mesh or screening... 50-74% of homes have non-combustible ventilation soffits with mesh or screening... < 50% of homes have non-combustible ventilation soffits with mesh or screening.... Recommended Mitigation Strategies.. ◻.; Clean vents to keep them free of debris, allowing them to keep embers out while allowing air flow for ventilation... ◻.; Enclose or box-in eaves with non-combustible materials such as metal, cement board or stucco... ◻.; Install a 1/8 inch metal screen behind roof vents...... Siding.. → > 75% of homes have non-combustible siding... 50-74% of homes have non-combustible siding... < 50% of homes have non-combustible siding.... Recommended Mitigation Strategies.. ◻.; Keep landscaping materials and vegetation away from combustible siding... ◻.; Create 5-foot non-combustible area (Zone 0) around house... ◻.; Replace with noncombustible siding when possible......

Underskirting.. > 75% of homes have skirting underneath raised floors/decks... 50-74% of homes have skirting underneath... → < 50% of homes have skirting underneath.... Recommended Mitigation Strategies.. ◻.; Remove combustible vegetation and leaf litter... ◻.; Spread gravel or other non-combustible material under the deck... ◻.; Screen in the bottom of the deck with metal 1/8-inch screening... ◻.; Separate wooden fences from the house with a stone or metal barrier...... Wooden Attachments.. > 75% of homes have NO Wooden Attachments... 50-74% of homes have NO Wooden Attachments... → < 50% of homes have NO Wooden Attachments.... Recommended Mitigation Strategies.. ◻.; Â Maintain debris-free decks (e.g. remove ignitable furniture, planters and covering propane grills, especially during high fire danger periods)... ◻.; Consider disconnecting fences from structures, or replacing materials directly attached to structures with fire resistant materials... ◻.; Be aware that wooden attachments can act as a fuse to the structure...... Building Setback.. → Not applicable... Greater than or equal to 30 feet from slope... Less than 30 feet from slope.... Recommended Mitigation Strategies.. ◻.; N/A...... Propane.. > 30 feet from the house and surrounding vegetation maintained... Fewer than 30 feet from the house and/or surrounding vegetation not maintained... → N/A.... Recommended Mitigation Strategies.. ◻.; N/A......

| Electric Utilities.. Electric Underground... → Electric Overhead drop maintained... Electric Overhead drop not maintained.... Recommended Mitigation Strategies.. ◻.; Â Keep vegetation pruned and mowed around electric cabinets... ◻.; Place non-flammable mulch (rock, stone) around base of electrical cabinets... ◻.; Plant less flammable bushes and shrubs around electrical cabinets...... Non-Combustible Zone 0.. → > 75% of homes/outbuildings have adjacent 5-ft non-combustible zone... 50-74% of homes/outbuildings have adjacent 5-ft non-combustible zone... < 50% of homes/outbuildings have adjacent 5-ft non-combustible zone.... Recommended Mitigation Strategies.. ◻.; Remove flammable materials and Create 5-foot non-combustible area (Zone 0) around house...... |
| --- |
|  |  | COMMENTS. |  |

## Voltaire

|  |  | Community Wildfire Risk Assessment. |  |
| --- | --- | --- | --- |
|  |  | Total Assessed Rating. |  |  |  |  |  |  |  |  |
|  |  | 114 - High. |  |  |  |  |  |  |  |  |
|  |  | Suppression Rating. |  |  |  |  |  |  |  |
|  |  | Low Hazard. |  |  |  |  |  |
|  |  | Surrounding Environment Rating. |  |  |  |  |  |  |  |
|  |  | High Hazard. |  |  |  |  |  |
|  |  | Structures Rating. |  |  |  |  |  |  |  |
|  |  | Moderate Hazard. |  |  |  |  |  |
|  |  | Fire Protection District. |  |  |  |  |  |  |  |
|  |  | Carson City Fire. |  |  |  |  |  |
|  |  | Fireshed(s). |  |  |  |  |  |  |  |
|  | Carson Valley-Carson River. |  |  |  |  |  |
|  | Community Information. |  |  |  |  |  |  |  |
|  |  | Latitude. | 39° 7' 33". |  |  |  |  |  |
|  |  | Longitude. | -119° 46' 29". |  |  |  |  |  |
|  |  | Dwelling Units. | 50. |  |  |  |  |  |
|  |  | Size. | 131.52 acres. |  |  |  |  |  |
|  |  | Community Type. | Residential/Commercial. |  |  |  |  |  |
|  | Assessed By:. | Scott OBrien. |  |  |  |  |  |
|  |  | Assessment Date:. | 03-27-2023. |  |  |  |  |  |

|  | SUPPRESSION ASSESSMENT.. Ingress and Egress.. → 2 or more roads in/out with NO response/evacuation complexity... 2 or more roads in/out with SLIGHT response/evacuation complexity... 2 or more roads in/out with MODERATE/HIGH response/evacuation complexity... One road in and out (entrance and exit is the same).... Recommended Mitigation Strategies.. ◻.; Keep community ingress/egress open and maintained (cleared of vegetation)... ◻.; Develop community plan for evacuation routes, safe zones, staging areas... ◻.; If community is gated, develop evacuation plan and ensure emergency responder access... ◻.; Ensure residents know their closest exit in case of emergency... ◻.; Ask Local Fire Department about Ready, Set, Go!...... Road Width.. Road width is > 24 feet... → Road width is > 20 feet and < 24 feet... Road width is < 20 feet.... Recommended Mitigation Strategies.. ◻.; Keep shoulders of road clear for emergency vehicle use at all times... ◻.; Consider providing pull-offs every 100 yards...... Road Accessibility.. → Surfaced road... Non-surfaced road, grade less than or equal to 5%... Non-surfaced road, grade greater than 5%... Non-maintained dirt road.... Recommended Mitigation Strategies.. ◻.; Ensure that road maintenance plan is in place...... |
| --- | --- |

|  | Secondary Road Terminus.. → Roads ends in a cul-de-sac, diameter > 100 feet... Roads ends in a cul-de-sac, diameter < 100 feet... Dead end roads <200 feet long... Dead end roads >200 feet long.... Recommended Mitigation Strategies.. ◻.; Maintain unobstructed access into cul-de-sacs... ◻.; Ensure cul-de-sacs are free of vehicles and/or other items...... Street Signs.. → Present throughout, lettering 4 inches high, non-flammable and reflective... Inconsistent throughout, lettering 4 inches high, non-flammable and reflective... Present or inconsistent but wooden, non-reflective, or lettering less than 4"... Not present.... Recommended Mitigation Strategies.. ◻.; Keep street signs visible and clear of vegetation and fine fuels...... Driveways.. → Average driveway allows access to homes... Average driveway restricts access to homes.... Recommended Mitigation Strategies.. ◻.; Maintain driveway access and clearance...... Water Supply.. → Pressurized hydrants spaced less than 1000 feet apart... Pressurized hydrants spaced more than 1000 feet apart... Dry Hydrant(s) / Draft available within the community... Other accessible sources within community (pond, lake, etc.)... Water sources located within 4 miles of community (incl heli dip sites)... No water sources in or within 4 miles of the community.... Recommended Mitigation Strategies.. ◻.; Ensure hydrants and water sources are marked, accessible and properly maintained... ◻.; Keep hydrants clear of obstructions and vegetation...... |
| --- | --- |

|  | Geographic Features.. No notable geographical features present to hinder fire suppression... → Suppression efforts hindered by geographical features (e.g. hazardous terrain).... Recommended Mitigation Strategies.. ◻.; Ensure emergency responders are aware of local geographic features that can hinder fire suppression efforts; consider pre-suppression plan...... Local Response Resources.. → 5 mi. or less from Agency with Response Authority (Staffed FD)... 5 mi. or less from Agency with Response Authority (Mixed Staff/VFD)... 5 mi. or less from Agency with Response Authority (VFD)... > 5 mi. from Agency with Response Authority FD.... Recommended Mitigation Strategies.. ◻.; Establish and maintain contact with the closest Fire Department; consider pre-suppression plan... ◻.; Be aware of the importance of early detection and reporting of any emergency...... Community Organization/Governance.. GID present; HAS structure for sustained fire prevention and mitigation... HOA present; HAS structure for sustained fire prevention and mitigation... → Municipal govt present; HAS structure for sustained fire prevention and mitigation... GID present; LACKS structure for sustained fire prevention and mitigation... HOA present; LACKS structure for sustained fire prevention and mitigation... Municipal govt present; LACKS structure for sustained fire prevention and mitigation... Lacks any structure for sustained fire prevention and mitigation.... Recommended Mitigation Strategies.. ◻.; Host a Community Education Event at least once a year; Become a Firewise USA® Site... ◻.; Complete Community Risk Mitigation Project(s) as identified by Community Action Plan...... |
| --- | --- |

|  | SURROUNDING ENVIRONMENT ASSESSMENT.. Predominant Vegetation.. Light (grass)... → Medium (brush)... Heavy (timber, overgrown sage, Pinyon/Juniper with dead/down, etc)... Extreme / Slash (Any Combination of contiguous Light, Medium, Heavy).... Recommended Mitigation Strategies.. ◻.; Consider removal of ladder fuels that allow fire to climb from lower to higher vegetation... ◻.; Trim tree canopies regularly to keep their branches a minimum of 10' from structures and other trees... ◻.; Leave 30 feet between clusters of two to three trees, or 20 feet between individual trees... ◻.; Prune trees 6-10 feet from the ground...... Defensible Space.. > 75% of homes meet criteria in Zone 0, 1 & 2... 50 to 75% of homes meet criteria in Zone 0, 1 & 2... < 50% of homes meet criteria in Zone 0, 1 & 2 - Light fuels amongst structures... → < 50% of homes meet criteria in Zone 0, 1 & 2 - Moderate fuels amongst structures... Fuels heavy/extreme amongst structures & other urban hazards/materials are present.... Recommended Mitigation Strategies.. ◻.; Be aware of the risks from falling embers in relation to nearby fuels and defensible space... ◻.; Mow lawns regularly... ◻.; Water grass, plants, trees and mulch regularly... ◻.; Create a spacing of 30 feet between tree crowns... ◻.; Create a non-combustible area (zone 0) within 5 feet of your home, using non-flammable landscaping materials... ◻.; Remove dead vegetation from under the deck and within 10 feet of the house; stack firewood away from structures... ◻.; ConsiderÂ xeriscaping... ◻.; Plant a mixture of deciduous trees (e.g. oak and maple) and coniferous trees (e.g. pine)... ◻.; Create fuel breaks like driveways and gravel walkways...... |
| --- | --- |

|  | Structure-to-Structure Ignition.. No Possible Structure-to-Structure Ignition... → Possible Structure-to-Structure Ignition.... Recommended Mitigation Strategies.. ◻.; Work with neighbors to remove/prune vegetation between houses to mitigate structure-to-structure ignition risk; consder non-combustible fencing 5 feet from structure... ◻.; Consider use of sprinkler systems to keep vegetation moisture levels up... ◻.; Replace flammable roofs, siding, soffits, etc. with nonflammable when possible...... Slope.. Slope 0% - 5%... → Slope 6 % - 10%... Slope 11% - 30%... Slope > 31%.... Recommended Mitigation Strategies.. ◻.; N/A...... Vegetation on Electric Transmission Lines.. No above ground electric transmission lines present... → Above ground electric transmission lines are maintained... Above ground electric transmission lines are NOT maintained.... Recommended Mitigation Strategies.. ◻.; Know who to call should there be a problem with electric lines in community...... Topographical Features.. No topographical features adversely affect wildland fire behavior... → Topographical features adversely affect wildland fire behavior (box canyons, chimneys, etc.).... Recommended Mitigation Strategies.. ◻.; Maintain situational awareness of fire danger in your area, as local topographical features can adversely affect wildland behavior...... |
| --- | --- |

|  | Adjacency to Wildlands.. Not adjacent to wildlands with accumulated fuels... → Adjacent to wildlands with accumulated fuels.... Recommended Mitigation Strategies.. ◻.; When possible, install firebreaks and reduce fuel loads around community boundary to reduce risk from adjacent wildlands; Work with neighboring land owners...... Undeveloped Lots with Restricted Access and/or Not Maintained.. Fewer than 10% of lots are undeveloped... → 10% to 30% of lots are undeveloped... 31% to 50% of lots are undeveloped... Greater than 51% of lots are undeveloped.... Recommended Mitigation Strategies.. ◻.; Provide Living with Fire/Firewise construction guidelines to developers /owners... ◻.; Consider developing covenant restrictions, if applicable...... |
| --- | --- |

STRUCTURES ASSESSMENT.. Roofing Materials.. → > 75% of homes have metal, tile or class A asphalt or fiberglass shingles... 50 to 75% of homes have metal, tile or class A asphalt or fiberglass shingles... < 50% of homes have metal, tile or class A asphalt or fiberglass shingles.... Recommended Mitigation Strategies.. ◻.; Use fire-resistant roofing material such as metal, tile or Class A shingles... ◻.; Inspect for and address gaps in roofing that can expose roof decking or supports... ◻.; Place angle flashing over openings between the roof decking and fascia board...... Debris on Roof and/or Gutters.. → No... Yes.... Recommended Mitigation Strategies.. ◻.; Clear branch, leaf-litter and other debris from roof and gutters regularly... ◻.; Prune tree limbs away from roof...... Ventilation and Soffits.. → > 75% of homes have non-combustible ventilation soffits with mesh or screening... 50-74% of homes have non-combustible ventilation soffits with mesh or screening... < 50% of homes have non-combustible ventilation soffits with mesh or screening.... Recommended Mitigation Strategies.. ◻.; Clean vents to keep them free of debris, allowing them to keep embers out while allowing air flow for ventilation... ◻.; Enclose or box-in eaves with non-combustible materials such as metal, cement board or stucco... ◻.; Install a 1/8 inch metal screen behind roof vents...... Siding.. > 75% of homes have non-combustible siding... → 50-74% of homes have non-combustible siding... < 50% of homes have non-combustible siding.... Recommended Mitigation Strategies.. ◻.; Keep landscaping materials and vegetation away from combustible siding... ◻.; Create 5-foot non-combustible area (Zone 0) around house... ◻.; Replace with noncombustible siding when possible......

Underskirting.. > 75% of homes have skirting underneath raised floors/decks... 50-74% of homes have skirting underneath... → < 50% of homes have skirting underneath.... Recommended Mitigation Strategies.. ◻.; Remove combustible vegetation and leaf litter... ◻.; Spread gravel or other non-combustible material under the deck... ◻.; Screen in the bottom of the deck with metal 1/8-inch screening... ◻.; Separate wooden fences from the house with a stone or metal barrier...... Wooden Attachments.. > 75% of homes have NO Wooden Attachments... → 50-74% of homes have NO Wooden Attachments... < 50% of homes have NO Wooden Attachments.... Recommended Mitigation Strategies.. ◻.; Â Maintain debris-free decks (e.g. remove ignitable furniture, planters and covering propane grills, especially during high fire danger periods)... ◻.; Consider disconnecting fences from structures, or replacing materials directly attached to structures with fire resistant materials... ◻.; Be aware that wooden attachments can act as a fuse to the structure...... Building Setback.. → Not applicable... Greater than or equal to 30 feet from slope... Less than 30 feet from slope.... Recommended Mitigation Strategies.. ◻.; N/A...... Propane.. > 30 feet from the house and surrounding vegetation maintained... Fewer than 30 feet from the house and/or surrounding vegetation not maintained... → N/A.... Recommended Mitigation Strategies.. ◻.; N/A......

| Electric Utilities.. Electric Underground... → Electric Overhead drop maintained... Electric Overhead drop not maintained.... Recommended Mitigation Strategies.. ◻.; Â Keep vegetation pruned and mowed around electric cabinets... ◻.; Place non-flammable mulch (rock, stone) around base of electrical cabinets... ◻.; Plant less flammable bushes and shrubs around electrical cabinets...... Non-Combustible Zone 0.. > 75% of homes/outbuildings have adjacent 5-ft non-combustible zone... → 50-74% of homes/outbuildings have adjacent 5-ft non-combustible zone... < 50% of homes/outbuildings have adjacent 5-ft non-combustible zone.... Recommended Mitigation Strategies.. ◻.; N/A...... |
| --- |
|  |  | COMMENTS. |  |